MAJOR THESIS

PERFORMANCE ANALYSIS OF PULSE POSITION MODULATION SYSTEMS WITH CODING SCHEMES FOR DEEP SPACE OPTICAL CHANNEL

Submitted in partial fulfillment of the requirement

for the award of degree of

###### MASTER OF ENGINEERING

IN

ELECTRONICS AND COMMUNICATION ENGINEERING

SUBMITTED BY

# JYOTI DARGAN

# DELHI UNIVERSITY ROLL NO.-4606

D.C.E. ROLL NO.- 15/E&C/03

UNDER THE GUIDANCE OF

Mr. RAJESH ROHILLA

DEPTT. OF ELECTRONICS AND COMMUNICATION

# DEPARTMENT OF ELECTRONICS & COMMUNICATION

DELHI COLLEGE OF ENGINEERING

BAWANA ROAD, DELHI-110 042

### CERTIFICATE

The major project dissertation entitled “performance analysis of pulse position modulation systems with coding schemes for deep space optical channel” represents the work carried out by Ms. Jyoti Dargan ( university Roll. No. 4606) in partial fulfillment of the requirement  for the award of M.E. degree in Electronics and Communication Engg.

This project has been carried out under my supervision and guidance. I, hereby certify that this thesis has not been submitted anywhere else for the award of a degree or a diploma to the best of my knowledge & belief and   approve it for submission to the institute for the award of the degree.

#### Rajesh Rohilla

Assistant Professor

Delhi College of Engg.,

Bawana Road,  Delhi.

##### ACKNOWLEDGEMENT

I would like to express my profound gratitude to my Project Guide Sh. Rajesh Rohilla, Asso. Professor, Deptt. Of Electronics and Communication for the guidance, inspiration and support during the course of the project. Without his invaluable support and able guidance the completion of this project would not have been possible.

I would also like to thank my family for the unconditional support provided by them. Without their encouragement , it would have been really difficult to produce this work.

I am also indebted to many helpful people and fellow colleagues at DCE, Delhi, who have helped me with valuable inputs and views. I owe a lot to the facilities provided at the D.C.E. that has helped immensely in the preparation of this project report.

JYOTI DARGAN

Department Of Electronics & Communication

Delhi College of Engineering,

Bawana Road, Delhi- 110 042.

##### ABSTRACT

The deep space optical communication system involves communication without the optical fiber, i.e. it is an unguided communication system for free space. Since it offers high capacity links for inter-satellite, inter-orbital or deep space mission communication applications, it has been the subject of a lot of significant research. This system typically employs the intensity modulation/ direct detection scheme. Deep space communication channels are characterized by very low SNRs and practically no bandwidth limitations. Since the deep space optical channel is not bandwidth-constrained but is power constrained due to the large distances involved, pulse position modulation is preferred choice of modulation scheme in spite  of its large bandwidth requirement, as it is very power efficient.

In this thesis, four variants of the PPM scheme have been explored, viz., simple PPM, Amplitude PPM, Overlapping PPM and multi-pulse PPM. Performance evaluations have been carried out for these PPM systems with various coding schemes viz., trellis coded Modulation, Product code, RSC Code and Turbo Code. Coding schemes have certain advantages to offer such as reduction in the required power level at the receiver, an increase in the data rate and an improvement in the BER performance vis-à-vis the uncoded case. Trellis Coded Modulation provides these advantages by merging modulation schemes with channel coding. Turbo codes are a special class of concatenated codes that provide excellent performance at low SNRs and are composed of two RSC encoders in parallel, separated by an interleaver. Furthermore, the basic concepts of concatenation, iteration and soft-decision decoding, which are necessary for implementing Turbo codes, can be understood using the product code.

##### LIST OF FIGURES

Fig. 1.1: IM/DD link modeled as LTI system

Fig. 2.1: Model of deep space optical communication system

Fig. 2.2: Block diagram of PPM receiver

Fig. 2.3: Structure of PPM decoder

Fig. 3.1: Symbol set for 4-PPM

Fig 3.2: Symbol set for 2x2-APPM

Fig.3.3: Symbol set for (5,2) 4-OPPM

Fig. 3.4: Symbol set for 4C2 MP-PPM

Fig.4.1: Set partitioning for a four-symbol PPM constellation

Fig. 4.2: Symbols j and l in the time domain for determining Manhattan     Distance

Fig. 4.3(a): Convolutional encoder and (b) Four state Trellis diagram

Fig. 4.4: Four-state Trellis diagram for performance analysis

Fig.4.5: Simulation model for determining BER

Fig. 4.6: BER vs SNR for the Trellis Coded PPM schemes.

Fig. 5.1: Product Code

Fig. 5.2: Simulation model for determining BER

Fig. 5.3: Simulation model for determining BER

Fig.5.4: BER vs SNR for OOK with the product code for different decoder              Iterations

Fig. 5.5: BER vs. SNR for PPM schemes with the Product code for decoder     Iterations

Fig. 6.1: Recursive systematic convolutional (RSC) code

Fig. 6.2: Four-state Trellis diagram for RSC code

Fig. 6.3: Parallel concatenation of two RSC codes

Fig. 6.4: Simulation model for determining BER

Fig. 6.5: Simulation model for determining BER

Fig. 6.6: BER vs. SNR for OOK with RSC code

Fig. 6.7(a): BER vs. SNR for PPM schemes with RSC code (hard viterbi     decoder)

Fig. 6.7(b): BER vs. SNR for PPM schemes with the RSC code (soft Viterbi	     Decoder)

Fig. 6.8: Simulation model for determining BER

Fig.6.9(a): BER vs. SNR for 4-PPM with Turbo code at different decoder     Iterations

Fig. 6.9(b): BER vs. SNR for 2x2-APPM with Turbo code at different        decoder iterations

Fig. 6.9(c): BER vs. SNR for 4-OPPM with Turbo code at different decoder iterations

Fig. 6.9(d): BER vs. SNR for 4C2 MP-PPM with Turbo code at different decoder iterations

Fig. 6.10: BER vs. SNR for PPM schemes with Turbo code for 5 decoder iterations.

Fig. 7.1: BER vs. SNR for 4-PPM with different coding schemes.

Fig. 7.2: BER vs. SNR for 2x2-APPM with different coding schemes.

Fig. 7.3: BER vs. SNR for (5,2) 4-OPPM with different coding schemes.

Fig. 7.4: BER vs. SNR for 4C2 MP-PPM  with different coding schemes.

##### LIST OF TABLES

Table 4.1: Intersymbol distances Djl for various PPM schemes

Table 4.2: Asymptotic coding gain for Trellis coded PPM schemes

Table 7.1: Relative rankings of  different codes for the four PPM schemes.

Table 7.2: SNR requirements of different codes for the four PPM schemes at BER of 10-3.

##### LIST OF SYMBOLS AND ACRONYMS

RF			Radio Frequency

IR			Infra Red

FSO			Free Space Optics

LOS			Line-of Sight

PSD			Power Spectral Density

IM			Intensity Modulation

DD			Direct Detection

APD			Avalanche Photodiode

OOK			On-Off-Keying

PPM			Pulse Position Modulation

APPM		Amplitude Pulse Position Modulation

OPPM		Overlapping Pulse Position Modulation

MP-PPM		Multi-Pulse Pulse Position Modulation

DPPM 		Differential Pulse Position Modulation

BER			Bit Error Rate

SNR			Signal to Noise Ratio

AWGN		Additive White Guassian Noise

ISI			Intersymbol Interference

TCM			Trellis Coded Modulation

dfree			Free Distance

LLR			Log Likelihood Ratio

RSC			Recursive Systematic Convolutional

PCCC		Parallel Concatenated Convolutional Codes

SCCC		Serially Concatenated Convolutional Codes

HCCC		Hybrid Concatenated Convolutional Codes

##### TABLE OF CONTENTS

Certificate										        1

Acknowledgement								        2

Abstract									         	        3

List of Figures								   	        4

List of Tables									        7

List of Symbols and Acronyms						        8

Table of Contents								      	      10

CHAPTER 1

- Introduction								  	      14

  - Types of Optical Channels					    	      14

    - Guided optical channels					      14

    - Unguided optical channels					      14

  - free space optical vs. microwave link				      15

  - Non Coherent optical transmission system			      16

    - Intensity modulation schemes				      16

    - Direct detection scheme					      17

  - Organization of the thesis						      17

CHAPTER 2

Deep Space Optical Communication System					      19

  - Deep space optical channel						      19

  - Motivation for using pulse position modulation			      20

  - Receiver structure for PPM						      21

  - Intersymbol interference						      22

  - Motivation for using coding schemes				      23

CHAPTER 3

- Description of Pulse Position Modulation Schemes			    25

  - Simple pulse pposition modulation					    25

  - Amplitude pulse position modulation				    26

  - Overlapping pulse position modulation				    27

  - Multi-pulse pulse position modulation				    28

CHAPTER 4

- Trellis Coded PPM Schemes							    31

  - Introduction								    31

  - Distance metrics							    32

    - |j-l| metric for 4-PPM					    32

    - Euclidean distance metric for 2x2-APPM		    32

    - Manhattan distance metric for (5,2)4-OPPM and

4C2MP-PPM						    33

  - Evaluation of asymptotic coding gain for Trellis coded

PPM  Schemes								    33

  - Performance analysis of Trellis coded PPM

Schemes									    36

  - Observation and conclusions						    38

#### CHAPTER 5

- Performance Evaluation with Product codes				    40

  - Introduction								    40

  - Analysis of OOK modulation with Product code		    40

  - Analysis of PPM schemes with Product code 			    41

  - Observations and conclusions					    43

#### CHAPTER 6

- Performance Evaluation with Turbo codes				    46

  - Introduction								    46

  - Performance analysis of OOK modulation with RSC code	    49

  - Performance analysis of PPM schemes with RSC code	    50

  - Observations and conclusions for RSC code			    51

  - Performance analysis of PPM schemes with Turbo code	    52

  - Observations and conclusions for Turbo code			    53

#### CHAPTER 7

- Comparative study of PPM systems with various codes		    62

  - The 4-PPM  System							    62

  - The 2x2-APPM system						    63

  - The (5,2) 4-OPPM system						    64

  - The 4C2MP-PPM system						    64

  - Overall observations and conclusions				    64

Scope for Future work								    70

#### References										    71

Appendix: A guide to the MATLAB and C++ codes in the CD	    73

CHAPTER-1

INTRODUCTION

Optical communication systems offer a viable alternative to radio frequency (RF) communication systems for indoor applications, as well as to microwave systems used in deep space communication. The main advantages of infrared (IR)/Optical communication systems over RF and microwave systems are:

- IR can support an extremely large bandwidth, whereas RF can support only a limited band width because of restricted spectrum availability and interference.

- IR technology can provide communication at very high bit rates of the order of a few Gbps as compared to tens of Mbps supported by RF.

- IR beam spread is much smaller as compared to the microwave beam spread. Hence, not only does the IR beam have a higher power density as compared to the  microwave beam, but also causes less interference with other signals.

# 1.1 TYPES OF OPTICAL CHANNELS

# 1.1.1 Guided Optical Channels

Wired communication through optical fibers which can be of three types

- Multi mode Step Index

- Multi mode Graded- Index

- Single mode Step-Index.

# 1.1.2 Unguided Optical channels

Wireless communication using a pair of free space optics transceivers, each aiming laser beams at the other, thus creating a full duplex communication link.

This line-of-sight FSO link essentially behaves the same as a fiber optic system. Instead of focusing the output of a semiconductor laser into a strand of optical fiber, the output is broadcast in a thin beam, aimed at the receiving unit. Free space optical communication can be categorized as:

FSO

Deep space optical communication                  Terrestrial communication

1. Inter-satellite link                                      1. Outdoor IR link

2. Inter-orbital link                                        2.  Indoor IR link

3. Interplanetary link                                     - directed/non- directed/ hybrid

-LOS/non-LOS.

1.2 FREE SPACE OPTICAL LINK vs. MICROWAVE LINK

The free space optical link differs from the familiar microwave link in that its performance is limited by the intrinsic quantum noise rather than the receiver thermal noise. The noise power spectral density (PSD) at the receiver is described by:

NB =                hv

Exp ( hv / kT ) – 1

Where h is the plank’s constant, k is the Boltzmann’s constant. ν is the operating frequency and T is the reference temperature.

Since hv<<kT at the microwave frequencies, the noise PSD from the above equation will approximately be kT. At the optical frequencies, energy of the photon is much greater than the thermal noise PSD, i.e. hv>>kT. As a consequence, thermal noise is a limiting factor at the microwave frequencies, but it may not be so at the optical frequencies.

This is not surprising. It is well known from thermodynamics that hv and kT are the two limiting forms of the noise PSD for an oscillator at the frequency v and in thermal equilibrium at the temperature T. When hv<<kT, thermal noise dominates and when hv>>kT, quantum noise dominates.

1.3 NON-COHERENT OPTICAL TRANSMISSION SYSTEM

In optical communication, the most commomnly used system is Intensity Modulation/Direct Detection (IM/DD)[1]. This is a non-coherent system, which is less complex to implement than a coherent one.

1.3.1 INTENSITY MODULATION SCHEMES

Intensity Modulation involves the use of On-Off Keying (OOK) and Pulse Position Modulation(PPM). A comparison between the two schemes is as follows:

- PPM is more complex to implement as it requires a high level of accuracy for slot synchronization.

- PPM is subject to bandwidth expansion.

- However, power requirement in the PPM is much less compared to OOK.

These tradeoffs lead to the following possibilities for the various optical channels:

- The guided optical fiber channel uses OOK mostly.

- The unguided deep space channel uses PPM.

- Both the outdoor and indoor channels use OOK when a low data rate is required, or PPM when a high data rate is required.

1.3.2 DIRECT DETECTION SCHEMES

Direct detection is obtained at the receiver using a photodiode (PIN/APD), which produces a photocurrent proportional to the optical power incident upon it. The modeling of IM/DD channel is as follows:

∞

y(t)= r ∫P (ζ)h(t- ζ) dζ + n(ζ)

Where the received photocurrent y(t) is the convolution of the transmitted optical power P(t) with the channel impulse response h(t), scaled by the photo detector responsivity r, plus an additive noise n(t), which is usually modeled as a white Guassian noise independent of P(t).

Furthermore, an IM/DD link is subject to an additional constraint that P(t) cannot be negative, since it describes the optical power.

rh(t)

P(t)                                                                                        y(t)

Optical Power                                                                   Photocurrent

n(t)

Signal-independent noise

Fig. 1.1: IM/DD link modeled as LTI system

1.4 ORGANIZATION OF THE THESIS

In this dissertation, fundamental theory, system modeling and performance evaluation of various PPM systems with coding schemes for the deep space optical channel have been presented.

- Chapter 2 describes the deep space optical channel, followed by the motivation for using PPM as the main choice of modulation scheme for this channel. A section on the role of coding schemes is also included.

- Chapter 3 is an overview of the various PPM schemes considered in this work along with comparisons in terms of bandwidth and power efficiency.

- Chapter 4 introduces three different distance metrics for the four PPM schemes. An evaluation of the asymptotic coding gains after merging these PPM schemes with Trellis Coded Modulation is carried out, followed by a comparative performance analysis in terms of BER vs. SNR.

- Chapter 5 describes the performance evaluation for both OOK modulation scheme and the four PPM schemes with Product code.

- In chapter 6, performance evaluation for both OOK modulation scheme and the four PPM schemes has been discussed with an RSC code. It is  followed by a comparative performance analysis for these PPM schemes with Turbo code, which is nothing but a parallel concatenation of two identical RSC codes separated by an interleaver.

- Chapter 7 concludes the thesis by making a comparative study of the four PPM systems with the various coding schemes. A brief section on the scope for future work has also been included.

##### CHAPTER 2

DEEP SPACE OPTICAL COMMUNICATION SYSTEM

2.1 DEEP SPACE OPTICAL CHANNEL

Deep space communication channels are characterized by very low SNRs and  practically no bandwidth limitations [7]. The transmitted power is usually obtained from on-board solar cells and, therefore, is typically limited to 20-30 W. The physical dimensions of the transmitting antenna are also quite limited and, therefore, its gain is limited too. The enormous distance between the transmitter and the receiver and lack of repeaters results in a very low SNR at the receiver. The channel noise can be characterized by a white Guassian random process. These channels are very well modeled as AWGN channels.

The deep space optical channel is unusual in that it combines the intensity modulation constraints of the Poisson photon counting channel with the additive white Guassian noise (AWGN) of the conventional channel [2]. The received signal can then be modeled as a Poisson process with rate λs(t) + λn(t). Where λs(t) is proportional to the instantaneous optical power of the received signal, and λn(t) is proportional to the power of the background light.

For the analysis however, the AWGN channel model has been assumed, as shown in Fig 2.1. This is essentially the same as the noisy photon counting channel in the limit [2]. Working with the AWGN channel has its advantages, for it is not only accurate at a high bit rate, but makes the analysis simpler under a reasonable approximation.

Encoder

Laser

Modulation

Detector

Demodulator

Decoder

Input bit stream

Light pulses

Slot count

Decoded bit stream

AWGN

Fig 2.1 : Model of deep space optical communication system

2.2 MOTIVATION FOR USING PULSE POSITION MODULATION

The deep space optical channel is not bandwidth-constrained, but is certainly power-constrained. This critical fact leads to the choice of PPM as the optimum signaling scheme in the deep space optical communication system design for several reasons:

- PPM is a highly power efficient scheme.

- Of the intensity modulation schemes, PPM is more suitable than its OOK counterpart for deep space application. Since a laser with a peak power large enough to survive the huge deep space loss is required, Q-switched lasers are typically used. However, current technology does not support toggling of this laser between ON and OFF state at the high rate, severely limiting the data rate that can be supported using OOK scheme.

- The OOK communication system may suffer a synchronization loss if the clock recovery subsystem encounters a sequence of zeros. On the other hand, PPM has at least one pulse necessarily in every symbol frame, benefiting the clock recovery process.

- Unlike OOK, an a priori knowledge of the signal or background radiation levels is not required to implement an optimum PPM receiver.

2.3 RECEIVER STRUCTURE FOR PPM

The PPM transmitter transmits the optical pulses in the appropriate time slots of the symbol duration, which propagate to the receiver over free space. The finite propagation time gives rise to a delay that is generally not known with great accuracy and further, may be varying with time as well.

As shown in fig.2.2, the front end of the PPM receiver is a conventional photo detector, connected to the clock recovery circuit. This circuit recovers the clock waveform from the received PPM signal to provide for slot synchronization. The main purpose of slot synchronization is to establish a one-to-one correspondence between the receiver clock and the transmitter clock following the propagation through the physical channel.

Photo

Detector

Clock

Recovery

Circuit

M-PPM

Decoder

Transmitted signal

Decoded signal

Fig. 2.2: Block diagram of PPM receiver

As shown in Fig. 2.3, the PPM decoder integrates the photo detector output signal over each slot period, and then applies the appropriate decision rule to the M integrated outputs, depending on the PPM scheme under consideration. Ni  is the number of photons counted in each slot.

The motivation for using a photon-counting receiver structure that it provides a very high gain of the order of 106.

Photo

Detector

Count Over

1st Slot

Count Over

2nd Slot

Count Over

Mth Slot

Ni

Ni

Ni

Decision

Rule

Decoded

Fig. 2.3: Structure of PPM decoder

2.4 INTERSYMBOL INTERFERENCE

The analysis of PPM generally assumes that the signaling set is orthogonal, whereby the rectangular pulse is considered (i.e., no ISI). However, in deep space direct detection applications, a large detector area is employed so as to collect more photons, thus increasing the no. of photon per information bit. Unfortunately, a larger detector size leads to a lower detector bandwidth [2], which tends to smear the observed pulses over several adjacent slots, thus destroying the orthogonality of the signaling set and resulting in severe ISI and an increased error probability.

Therefore the non-rectangular (Guassian ) pulse shape has been used throughout the analysis, to accommodate pulse spreading beyond the slot boundary (i.e., ISI).

2.5 MOTIVATION FOR USING CODING SCHEMES

Coding schemes give rise to one, or a combination, of the following advantages:

- Reduction in the required power level at the receiver.

- Increase in the data rate.

- Improvement in the BER performance.

In the process, however, there is a price to pay in terms of bandwidth expansion, due to the redundancies added as a part of the coding scheme itself. However, since bandwidth is not a major concern in the deep space optical channel, both block and convolutional coding schemes can be applied.

One of the interesting coding schemes is Trellis Coded Modulation (TCM)[3], which merges modulation schemes with channel coding. This technique provides all the aforesaid advantages of coding at no bandwidth expansion. A second advantage of this technique lies in its flexibility to accommodate numerous modulation schemes. TCM is dealt with in chapter 4.

The advantage of coding can also be obtained using the product code [5,7]. Furthermore, the basic concepts of concatenation, iteration, and soft-decision decoding, which are necessary for implementing Turbo codes, can be understood using this code. Product codes are discussed in chapter 5.

Turbo Codes [3,5,6,7] are a special class of concatenated codes where there exists an interleaver between two parallel or serial encoders. The existence of interleaver results in very large code word lengths with excellent performance, particularly at low SNRs. It is shown in the literature that using these codes, it is possible to get as close as 0.7 dB to the Shannon limit [3] at low SNRs. This provides an extremely strong motivation for studying this class of codes for the power-constrained deep space channel. Turbo codes, along with its constituent RSC code, are discussed in chapter 6.

CHAPTER 3

DESCRIPTION OF PULSE POSITION MODULATION SCHEMES

In this chapter, four variants of the PPM scheme have been considered along with examples of their four-symbol sets. In fact, these four symbol sets are what have been used for performance evaluation with various coding schemes in the rest of the thesis.

3.1 SIMPLE PULSE POSITION MODULATION

In M-PPM [1], each word of k bits is mapped onto one of M=2k symbols and transmitted through the channel. The M waveforms are given by

PppTc(t-iTc)                                  with i= 0,1,……,M-1

Where pTc(t) is a rectangular pulse with unit amplitude and chip duration Tc= Ts/M. The average power is related with the peak power by Pav= Pp/M

Example of 4-PPM

In 4-PPM, the symbol interval Ts= 2 Tb comprises of four subintervals (or chips) of equal duration Tc= Tb/2. The four symbols are shown in Fig. 3.1.

SYMBOL 1

SYMBOL 3

SYMBOL 2

SYMBOL 4

Fig. 3.1 : Symbol set for 4-PPM (Ts= 2 Tb,  Tc= Tb/2.  Tb is one bit interval)

3.2 AMPLITUDE PULSE POSITION MODULATION

In AxM-APPM[4], each symbol consists of M chips of duration Tc as in M-PPM. However, in addition, the pulse may take one of the A possible amplitude values. The AxM waveforms are given by

jPppTc(t-iTC)                        with j= 1,……..,A

and i= 0,1,……….,M-1

The average power is related with the peak power by Pav= Pp* (1+A)/2M.

Advantage: It is clear that going from M-PPM to AxM-APPM expands the signal set size by a factor of A without a bandwidth expansion since the duration of the elementary chips remains the same[4].

Example of 2x2-APPM

In 2x2-APPM, the symbol interval Ts= Tb comprises of two chips of equal duration Tc= Tb/2, as in 2-PPM. The difference is that the pulse may take one of two possible amplitude values, yielding four symbols as shown in fig. 3.2.

Symbol 1

Symbol 3

Symbol 4

Symbol 2

Fig. 3.2 Symbol set for 2x2- APPM (Ts= Tb,  Tc= Tb/2)

3.3 Overlapping Pulse Position Modulation

In (n,w) M-OPPM [4], the symbol interval Ts is divided into n chips of equal duration Tc. The information is conveyed by the position of a pulse of duration Tp=wTc. , starting in one of the M instants ti=(i-1)Tc with i=1,2,…..M. It is clear that M is related to n & w by M=n-w+1. Thus, the M waveforms allowed for M-OPPM symbols are given by

PppTp(t-iTC)					with i = 0,1,….M-1

Where pTp(t) is a rectangular pulse with unit amplitude and duration Tp=wTc. The average power is given by Pav=Pp*(w/n).

Criterion: To obtain efficient M-OPPM schemes [4], (n,w) must be chosen such that M is a power of 2. For example: (5,2), (6,3) and (7,4) for M=4 or (9,2), (10,3) for M=8.

Advantage: It can be easily observed that Q=n/w is the alphabet size of the Q-PPM signaling set with no overlap, and that by allowing overlap between pulses, the number of signals gets increased from Q to M[4]. This is of course true only when the pulse width of the M-OPPM signal is kept the same as that for Q-PPM.

Disadvantage: On the other hand, the extended signal set is no longer orthogonal, which in turn implies worse error probability performance for uncoded transmission systems.

Example of (5,2) 4-OPPM

In (5,2) 4-OPPM, the symbol interval Ts=2Tb comprises of 5 chips of equal duration Ts= 2Tb/5. The information word is conveyed by the position of a pulse of duration  Tp= 2Tc., starting at one of the four instants ti=(i-1)Tc with i= 1,2,3,4. The four symbols are shown in Fig.3.3.

Symbol 1

Symbol 3

Symbol 4

Symbol 2

Fig.3.3: Symbol set for (5,2) 4-OPPM (Ts= 2 Tb, Tc= 2Tb/5, Tp= 2 Tc  =4Tb/5)

3.4 Multi-Pulse Pulse Position Modulation

In MP-PPM[2], the laser is pulsed in p slots of one frame consisting of M slots. Thus, one of Q = MCP word patterns can be sent in one frame, e.g. (32,2) MP-PPM has 496 word patterns.

Furthermore, when L bits are transmitted in one frame, L is selected to satisfy the condition L= └ log2Q┘. Hence, in (32,2) MP-PPM, 28=256 word patterns out of 496 word patterns are used for transmitting 8 bits of information, which allows the flexibility of choosing the optimum symbol set. Also, the bit rate R is given by

R= (log22L)/(MTslot)= L/(MTslot)

Where Tslot is the slot duration.

Criterion: Optimum 2L symbol sets [4] minimize the symbol error probability from Q word patterns which can be selected from a larger set. Symbols with p slots pulsed in continuation are to be definitely included in an optimum 2L symbol set. The remaining symbols are then determined by trial and error using the properties of their distances with other symbols in the set.

Advantage: The number of word pattern increase on increasing the number of pulses in one frame where p≤M/2. When L bits are transmitted in one frame, M-PPM requires 2L whereas MP-PPM requires M slots. Thus, the number of slots in one frame of MP-PPM is less than that of M-PPM because 2L -M≥0. From equation (3 d), the slot duration Tslot increases on decreasing M, provided R is fixed. Furthermore, the transmission bandwidth is approximately measured by the inverse of the time slot duration.

Therefore, for the same frame duration and the same bit rate, decrease in number of the word patterns allows larger slot duration leading to reduction in the transmission bandwidth.

Example of 4C2 MP-PPM

In 4C2 MP-PPM, the symbol interval Ts=2Tb comprises of two chips of equal duration Tc=Tb/2. Two of the four chips are pulsed, yielding 4C2=6 possible symbols, which gives the flexibility to choose the optimum symbol set, comprising of the four symbols as shown in fig.3.4.

Symbol 1

Symbol 3

Symbol 4

Symbol 2

Fig.3.4: Symbol set for 4C2 MP-PPM (Ts= 2 Tb, Tc= Tb/2)

Although the four symbol PPM sets discussed in this chapter have been depicted through rectangular pulses, it is only for explanatory purposes. For the actual performance simulations carried out in chapters 4,5 and 6, Gaussian pulses have been used.

In the next chapter, these PPM schemes are merged with Trellis Coded Modulation, and an evaluation of the asymptotic coding gains as well as a comparative performance analysis is carried out.

CHAPTER 4

TRELLIS CODED PPM SCHEMES

4.1 Introduction

Trellis Coded Modulation (TCM) [3] can be used to overcome the impact of ISI caused by imperfect pulse shapes, as discussed in chapter 2. This is achieved by following Ungerboeck’s heuristic set partitioning methodology[3], whereby the minimum distance between two symbols selected from any one of the partitioned symbol sets gets increased from that between two symbols selected from the original symbol set. This ensures that there exists no overlap among the pulses.

A={1, 2, 3, 4}

B1={2, 4}

B0={1, 3}

Fig.4.1 : Set partitioning for a four symbol PPM constellation

As shown in fig.4.1, let A= {1, 2, 3, 4} be the set representing the four uncoded PPM symbols. This set can be divided into sets B0={1, 3} and B1={2 4} in accordance with Ungerboeck’s rules. The four symbol set could, in general, belong to any of the four PPM schemes. The difference, however, lies in the distance metric employed, whereby distance between a pair of symbols j and l selected from A will defer depending on the PPM scheme under consideration.

4.2 Distance Metrics

For the analysis, three different distance metrics have been used to accommodate the four PPM schemes.

4.2.1 │j-l│ Metric for 4-PPM

As per this metric, the distance between a pair of symbols j and l is simply │j-l│. Thus, a pair of symbols selected from either one of B1 or B0 is at a distance of two, whereas a pair selected from A is at a minimum distance of one. For eliminating the impact of pulse spreading, a minimum distance of two is sufficient for establishing orthogonality among the symbols in the set, i.e., a minimum distance of two among the symbols selected from either one of B1 or B0  ensures that there exists no overlap among the pulses in the set. Qualitatively, this holds true for the other distance metrics as well.

4.2.2 Euclidean Distance Metric for 2x2-APPM

As per this metric [3], the distance between a pair of symbols j and l is the Euclidean distance between the two on the constellation diagram of the signaling scheme under consideration. This is a common metric employed in TCM.

4.2.3 Manhattan Distance Metric for (5,2), 4-OPPM and 4C2 MP-PPM

This metric computes the manhattan distance between two symbols. For a two-pulse case, the Manhattan distance between symbols j and l is defined as

a = │ws- ys│+│we- ye│ 					(1a)

b = │xs- zs│+│xe- ze│					(1b)

Manhattan distance, Djl= ( a2 + b2)1/2	                    (1c)

ws     we         xs    xe

Symbol j

Symbol l

ys        ye         zs       ze

Fig.4.2: Symbols j and l in the time domain for determining Manhattan distance

In Table 4.1, intersymbol distances, by employing the appropriate distance metric for each PPM scheme, are given.

| Distance | 4-PPM | 2x2- APPM | (5,2) 4-OPPM | 4C2 MP-PPM |
| --- | --- | --- | --- | --- |
| D12 | 1 | √2 | 1.6 √2 | 2√2 |
| D13 | 2 | √5 | 3.2 √2 | 4√2 |
| D14 | 3 | 1 | 4.8 √2 | 4 |
| D21 | 1 | √2 | 1.6 √2 | 2√2 |
| D23 | 1 | 1 | 1.6 √2 | 2√2 |
| D24 | 2 | √5 | 3.2 √2 | 2√2 |
| D31 | 2 | √5 | 3.2 √2 | 4√2 |
| D32 | 1 | 1 | 1.6 √2 | 2√2 |
| D34 | 1 | 2√2 | 1.6 √2 | 4 |
| D41 | 3 | 1 | 4.8 √2 | 4 |
| D42 | 2 | √5 | 3.2 √2 | 2√2 |
| D43 | 1 | 2√2 | 1.6 √2 | 4 |

Table 4.1 : Intersymbol distances Djl for various PPM schemes

4.3 Evaluation of Asymptotic Coding Gain for Trellis coded PPM Schemes

For the analysis, a 2-bit shift register has been considered as the convolutional encoder of constraint length-two. Without loss of generality, the encoder takes one input bit at a time, as shown in Fig.4.3a. Correspondingly, a four state Trellis diagram is obtained, as shown in fig.4.3b. The diagram has two branches diverging from each node and two branches converging to each node. The solid branches on the diagram represent a transition made when the input is 0, while the dotted branches represent a transition made when the input bit is 1.

The four-symbol PPM schemes have been used as the mapping constellation for the branches of the diagram. For each Trellis coded PPM scheme, computer simulations have been carried out for determining the d2 free(coded) . In these simulations, all possible permutations of the symbol sets B0/B0*≡ {1, 3}/ {3, 1} and B0/B1*≡ {2, 4}/ {4, 2} have been mapped onto the nodes of the Trellis diagram. For each permutation, d2free(coded) has been determined.

For a particular Trellis coded PPM scheme, the minimum  d2free(coded)  obtained over all possible mappings has been retained along with the corresponding mapping. Thereafter, the asymptotic coding gain[3] is given by

Asymptotic coding gain = 10 log10{d2free(coded)/ d2free(uncoded) } dB

Where d2free(uncoded) is for the uncoded 2-PPM scheme, which is used as the benchmark for each Trellis coded PPM scheme. Its value, therefore , depends on the distance metric employed for the Trellis Coded scheme under consideration.

Sl

So

(a)

Possible Mapping

(B0/B1)	00

(B1/B*1)	01

(B0/B0*)	10

(B1/B1*)	11

time				(b)

Fig. 4.3 (a): Convolutional encoder and (b) Four-state Trellis diagram

In table 4.2, the results for d2free(coded), d2free(uncoded) and the corresponding asymptotic coding gain for the Trellis coded PPM schemes are given

| # PPM scheme | d2free (uncoded) | d2free(coded) | Asymptotic coding gain (dB) |
| --- | --- | --- | --- |
| 4-ppm | 1.0 | 5.0 | 6.9897 |
| 2x2- APPM | 5.0 | 6.0 | 0.7918 |
| (5,2) 4- OPPM | 4.0 | 25.6 | 8.0618 |
| 4C2 MP-PPM | 4.0 | 16.0 | 6.0206 |

Table 4.2: Asymptotic gain for trellis coded PPM schemes

# 4.4 PERFORMANCE ANALYSIS OF TRELLIS CODED PPM SCHEMES

It has been found the results of the previous section that the mapping in fig. 4.4 is optimum for obtaining minimum d2free(coded) across all the Trellis coded PPM schemes. Using this Trellis diagram, performance analysis for each Trellis coded PPM scheme in terms of BER at different SNRs has been carried out through computer simulations.

Optimum mapping

(3,1)	00

(2,4)	01

(1,3)	10

(2,4)	11

Time

Fig. 4.4: Four –state Trellis diagram for performance analysis

The simulation process has been devised as follows. A bit stream of burst length 20 is randomly generated, forming the input to the Trellis encoder. Depending on the choices of PPM scheme, the encoder outputs a Trellis coded PPM symbol stream of the same length, which is then corrupted by AWGN. This corrupted stream (at a particular SNR) is demodulated, one symbol at a time, by the appropriate maximum likelihood decision module. The demodulated symbols form the input to the Viterbie decoder [3], which yields the decoded output bit stream of burst length 20.

The output bit stream is compared with the input bit stream, and the number of bits in error is accrued onto the total number of bits in error. The block diagram for one iteration of the simulation process is shown in fig. 4.5. Each point on the BER performance curve has been obtained using 103 iterations of this simulation process. Thus, at a particular SNR, the BER is given by

( total number of bits in error)

BER= ___________________________

(burst length) x (number of iterations)

It can be written as

BER= (Total no. of bits in error)/ (20 x 103)

BER= (Total no. of bits in error)/ (2 x 104)

PPM scheme                         AWGN

Random

Bit stream

generator

Trellis

Encoder

Input bit stream                                         Symbol stream               Corrupted

Symbol stream

Photon

counter

ML decision

module

Viterbie

decoder

Output stream                                         Symbol stream                                                    slot count

PPM scheme

Fig.4.5  Simulation modulation for determining BER

The BER performance at various SNRs for the Trellis coded PPM schemes are shown in graphical form in fig. 4.6.

# 4.5 Observations and conclusions

Following are the observations and conclusions made from the simulation results presented in fig. 4.6:

- A crossover point is observed in the BER curves of 4-PPM  and 2x2- APPM, which occurs at SNR of about 1 dB. For SNR≥1 dB, BER performance of 4-PPM is better, while for SNR < 1 dB, BER performance of 2X2- APPM becomes better.

- There is a crossover point in the BER curves of (5,2) 4-OPPM and 2x2-APPM. In this case, the crossover point occurs at SNR of about 5dB. For SNR≥5dB of (5,2) 4-OPPM is lower, while for SNR<5 db, BER of 2x2-APPM is lower.

- BER performance of  4C2 MP-PPM is worse than the other PPM schemes for all SNRs considered.

Finally, it can be concluded that for SNR≥ 1dB, BER performance of 4-PPM is the best, while for SNR< 1dB, BER performance of 2x2-APPM becomes the best.

In this chapter, evaluation of the asymptotic coding gains for the Trellis coded PPM schemes as well as their performance analysis has been carried out. In the next chapter, the basic concept of concatenation, iteration and soft-decision decoding are explored using Product code.

## CHAPTER 5

# PERFORMANCE EVALUATION WITH PRODUCT CODE

# 5.1 Introduction

A product code [5,7] is a simple example of a concatenated code. Its structure encompasses two separate encoding steps, horizontal and vertical, as depicted in fig. 5.1.  The configuration can be described as a data array  made up of k1 rows and k2 columns. The k1 rows contain code words made up of  k2 data bits and n2-k2 parity bits. Thus, each row represents a code word from an (n2, k2) code. Similarly, the k2 columns contain code words made up of  k1 data bits and n1-k1 parity bits. Thus, each column represents a code word from an (n1, k1) code. The various portions of the structure are labeled d for data, ph for horizontal parity, and pv for vertical parity.

The product decoder [5] is an iterative soft input/soft output decoder. The blocks labeled Leh and Lev contain the extrinsic Log-Likelihood Ratio (LLR) values [5] learnt respectively from the horizontal and vertical decoding steps, which together constitute one decoder iteration. The number of decoder iterations, Nd, can be varied prior to taking a hard decision, and for an iterative decoder in general, a performance improvement is expected with an increase in the number of decoder iterations.

k2          		n2-k2

column   		  column

d

Ph

Leh

k1  rows							Extrinsic horizontal

Pv

n1-k1 rows

Lev

Extrinsic Vertical

Fig. 5.1: Product code

5.2 Analysis of OOK Modulation With Product  Code

For the analysis, a Product code called the two-dimensional Single-parity code[5] has been considered with the parameters k1=2, n1- k1=1, k2=2 and n2-k2=1. Performance analysis for OOK with this code in terms of BER at different SNRs has been carried out for Nd = 1, 2, 3 and 4 through computer simulations.

The simulation process has been devised as follows. A bit stream of burst length 4 is randomly generated, forming the input to the Product encoder. The encoder outputs a data-plus-parity bit sequence of length 8, which is then corrupted by AWGN. This corrupted sequence (at a particular SNR) is passed into the Product decoder, which yields an output bit stream of length 4.

The output bit stream is compared with the input bit stream, and the number of bits in error is accrued onto the total number of bits in error. The block diagram for one iteration of the simulation process is shown in fig. 5.2.

Random bit

Stream generator

Product

Encoder

Input bit stream				OOK

AWGN

Product

Decoder

Soft Values				Output bit 											Stream

Nd

Fig. 5.2: Simulation model for determining BER

5.3 Analysis of PPM Schemes with Product Code

For the analysis, the same product code as above has been considered. Performance analysis for four-symbol PPM schemes with this code in terms of BER at different SNRs has been carried out for Nd = 2 through computer simulations.

In this case also, a bit of burst length 4 is generated as an input to the Product encoder. The encoder generates a data-plus-parity bit sequence of length 8, which is modulated, two bits at a time, using one of the four four-symbol PPM schemes. This PPM symbol stream of length 4 is then corrupted by AWGN. The corrupted stream (at a particular SNR) is demodulated, one symbol at a time, by the appropriate maximum likelihood decision module. The demodulated symbols form the input to the product decoder, which yields the decoded output stream of burst length 4.

The block diagram for one iteration of the simulation process is shown in fig. 5.3.

Random bit

Stream generator

Product

Encoder

PPM modulator

Random               Input bit

PPM

demodulator

Product

decoder

Stream

AWGN

Output bit stream					Hard values

Nd

### Fig. 5.3: Simulation model for determining BER

5.4 Observations and Conclusions:

For both OOK and PPM schemes, each point on the BER performance curve has been obtained using 104 iterations of the respective simulation process. Thus, at a particular SNR, the BER is given by

BER = ( Total no. of bits in error)/ ((burst length) x (no. of iterations))

- OOK with Product Code

The BER performance of various SNRs for OOK with the Product Code at different decoder iterations is shown in graphical form in fig. 5.4.

Following are the observations and conclusions are made from the simulation results presented in fig. 5.4:

- Compared to the uncoded case, BER performance improves on using OOK with the product code for all SNRs considered.

- However, as we increase the no. of product decoder iterations, BER performance improvement saturates at 2 decoder iterations.

- PPM schemes with Product Code

It can be considered from the previous subsection that two is the optimum no. of decoder iterations for the Product code. The BER performance at various SNRs for PPM schemes with the product code using 2 decoder iterations is shown in graphical form in fig. 5.5.

Following observations and conclusions are made from the simulation results presented in fig. 5.5:

- A crossover point is observed in the BER curves of (5,2) 4-OPPM  and 2x2-APPM which occurs at about 5.2 dB. is better.

- BER performance of 4-PPM is better than the other PPM schemes for all SNRs considered.

In this chapter, the basic concepts of concatenation, iteration, and soft-decision decoding have been explored using Product code. This iterative decoding is similar to the process used when solving a crossword puzzle. Using the row codes, we can come up with the best values and then, using the column codes, improve these guesses. This process can be repeated in an iterative fashion, improving the quality of the guess in each step. These ideas are applied to the implementation of Turbo codes which is discussed in the next chapter.

###### CHAPTER 6

# PERFORMANCE EVALUATION WITH TURBO CODE

# 6.1 Introduction

# Turbo codes [3,5,6,7], are also known as parallel concatenated convolutional codes (PCCCs), offer excellent error performance near the Shannon capacity limit with reasonable decoding complexity. Concatenated coding schemes are effective in dealing with bursts and random errors and at rates less than channel capacity, probability of error decreases exponentially, while decoding complexity increases only algebraically. High coding gains can be obtained with concatenated codes and with Turbo codes very high coding gains are achieved.

# Turbo encoder comprises of two or more recursive systematic convolutional (RSC) encoder [3,5,6,7], which may be identical or different, that are preceded by different interleavers and whose output bits are concatenated in parallel. A Turbo decoder consists of the corresponding decoders with a de- interleaver in between to modify error patterns in the received sequence. It employs iterative decoding and is soft input/soft output. The output of each component decoder contains three terms[5]: systematic information generated by the code information bit, extrinsic information generated by the code parity bit and a priori information generated by the other decoder. The extrinsic information is exchanged between the two component decoders.

For the analysis however, a Turbo encoder comprises of two identical rate ½ RSC encoders in parallel and separated by a random interleaver, has been considered. The decoding process [5,6] is based on the iterations of the maximum a posteriori (MAP) algorithm applied to each constituent code.

The RSC encoder used to build this Turbo encoder is depicted in fig. 6.1, itself is depicted in fig. 6.3.

The interleaver permutes the data in a random fashion, providing better error performance than the familiar block interleaver[5]. Furthermore, the encoded bit sequence is punctured by allowing vk to take up v1k and v2k alternately, where k is the sequence index. This increases the overall code rate from 1/3 to ½.

(dk)											    (uk)

ak

ak-1

ak-2

(vk)

Fig. 6.1: Recursive systematic convolutional (RSC) code

State

00

10

01

11

Input bit 0

Input bit 1

time

Fig. 6.2: Four state Trellis diagram for RSC code

(dk)										(uk)

ak

ak-1

ak-2

2

(v1k)

Interleaver

ak

ak-1

ak-2

(vk)

(v2k)

Fig. 6.3: Parallel Concatenation of two RSC codes

# 6.2 PERFORMANCE ANALYSIS OF  OOK WITH RSC CODE

To begin with, performance analysis using OOK with the RSC code (Fig. 6.1) in terms of BER at different SNRs has been carried out through computer simulations.

The simulation process has been devised as follows. A bit stream of burst length 20 is randomly generated, forming the input to the RSC encoder. The encoder outputs a Trellis coded bit sequence of length 40, which is then corrupted by AWGN. This corrupted sequence( at a particular SNR) is passed into the viterbi decoder, which yields an output bit stream of length 20.

The output bit stream is compared with the input bit stream, and the no. of bits in error is accrued onto the total no. of bits in error. The block diagram for one iteration of the simulation process is shown in fig. 6.4.

Random bit stream

Generator

RSC

Encoder

Input bit				 OOK

stream

Viterbi

Decoder

AWGN

Output

bit stream				          soft values

Fig. 6.4: Simulation model for determining BER

It is important to note that  for this analysis, performance using both the hard as well as the soft Viterbi decoders has been done. An 8-level quantizer (three bits/ component) has been employed for the soft Viterbi decoder.

6.3 PERFORMANCE ANALYSIS OF PPM SCHEMES WITH RSC CODE

For the analysis, the same RSC code as above has been considered. Performance analysis using four-symbol PPM schemes with this code has been carried out through computer simulations.

The simulation process has been devised as follows. A bit stream of burst length 20 is randomly generated, forming the input to the RSC encoder. The encoder  outputs a Trellis coded bit sequence of length 40, which is modulated two bits at a time, using one of the four four-symbol PPM schemes. The PPM symbol stream of length 20 is then corrupted by AWGN. This corrupted sequence( at a particular SNR) is demodulated, one symbol at a time, by the appropriate maximum likelihood decision module. The demodulated symbols form the input to the viterbi decoder, which yields the decoded output bit stream of length 20.

The block diagram for one iteration of the simulation process is shown in fig. 6.4. Once again, the analysis has been carried out using both the hard and the soft decoders.

Input bit

Random bit

Stream generator

RSC

Encoder

PPM

modulator

PPM

demodulator

Viterbi

decoder

Stream

AWGN

Output bit

Stream

Hard

Values

Fig. 6.5: Simulation model for determining BER

# 6.4 OBSERVATION AND CONCLUSIONS WITH RSC CODE

For both OOK and PPM schemes, each point on the BER performance curve has been obtained using 103 iterations of the respective simulation process. Thus, at a particular SNR, the BER is given by

BER= (Total no. of bits in error)/((burst length)x(number of iteration))

It can be written as

BER= ( Total number of bits in error)/ (20 x 103)

= (Total number of bits in error)/ (2 x 104)

a) OOK with RSC code

The BER performance at various SNRs for OOK with the RSC code using both the hard and the soft Viterbi decoder is shown as graphical form in Fig. 6.6.

Following observations and conclusions are made from the simulation results presented in Fig. 6.6:

- Compared to the uncoded case, BER performance improves on using OOK with the RSC code for all SNRs considered.

- The improvement is greater for the RSC code using soft Viterbi decoder.

In fact, this result comes as no surprise simce it is known from the literature that the difference between the performance of soft-and hard decision decoding is roughly 2 dB for an AWGN channel [7].

# (b)  PPM Schemes with RSC code

The BER performance for PPM schemes with the RSC code for the hard Viterbi decoder is shown in fig. 6.7(a) and for the soft Viterbi decoder in fig. 6.7.(b)

Following observations and conclusions are made from the simulation results presented in figs. 6.7(a) and (b):

- The results are almost identical for the two cases. This is to be expected since the output of the PPM demodulator, which forms the input to the Viterbi decoder, consists of hard values. Therefore, no improvement is to be expected on replacing the hard Viterbi decoder with the soft one.

- In both cases BER performance of 4-PPM is the best, while BER performance of 2x2-APPM is the worst for all SNRs considered.

6.5 PERFORMANCE ANALYSIS OF PPM SCHEMES WITH TURBO CODE

Performance analysis using four symbol PPM schemes with the Turbo code (Fig. 6.3) has been carried out through computer simulations.

The simulation process has been devised as follows. A bit stream of burst length 20 is randomly generated, forming the input to the Turbo encoder. The encoder outputs a coded bit sequence of length 40, which is modulated, two bits at a time, using one of the four four-symbol PPM schemes. This PPM symbol stream of length 4 is then corrupted by AWGN. The appropriate maximum likelihood decision module. The demodulated symbols form the input to the Turbo decoder [5,6], which yields the decoded output bit stream of burst length 20.

The block diagram for one iteration of the simulation process is shown in fig. 6.8.

Input bit

Random bit

Stream generator

RSC

Encoder

PPM

modulator

PPM

demodulator

Viterbi

decoder

Stream

AWGN

Output bit

Stream					Hard Value

Fig. 6.5: Simulation model for determining BER

# 6.6 Observations and conclusions with Turbo code

Each point on the BER performance curve has been obtained using 103 iterations of the simulation process. Thus, at a particular SNR, the BER is given by

BER = ( Total number of bits in error)/ (20 x 103)

= ( Total number of bits in error)/ ( 2 x 104)

The BER performance with the Turbo code at different decoder iterations is shown in graphical form for 4-PPM in fig. 6.9 (a), for 2 x 2- APPM in fig. 6.9(b), for (5,2) 4-OPPM in Fig. 6.9 (c) and for 4C2 MP-PPM in Fig. 6.9(d). In each of these, the uppermost curve corresponds to 1 decoder iteration, while the lowermost curve corresponds to 4 decoder iterations.

Following observations and conclusions are made from the simulation results presented in figs. 6.9(a), (b), (c) and (d):

- As the number of decoder iterations is varied from 1 to 4/5, the performance for each of the PPM schemes with the turbo code improves, but the improvement itself saturates within 5 iterations.

To make a comparative analysis of the four PPM schemes with the Turbo code, the curve corresponding to 5 decoder iterations is selected from each of the four cases, since that curve represents the best BER performance for each as per the analysis just done. This analysis is shown in graphical form in fig. 6.10.

It is observed from Fig. 6.10 that the curves for the four PPM schemes are more or less overlapping over the entire range or SNRs considered. Hence, it can be concluded that in the case of  Turbo codes, on one  PPM scheme performs either better or worse than the other, and each of them is equally acceptable. This is contrary to the results obtained for PPM schemes with any of the other coding schemes considered so far.

In this chapter, performance analysis with both RSC code and Turbo has been carried out. In the next chapter, a comparative study is made for PPM schemes with all the codes considered in the thesis.

## CHAPTER 7

# Comparative study of PPM Systems with Various Codes

In this chapter, a comparative study of the BER performance for the four-PPM schemes with all the codes considered in the thesis, viz., Trellis coded modulation, the Product code (at 2 decoder iterations), the RSC code and the Turbo code (at 4/5 decoder iterations) has been done.

# 7.1 The 4-PPM System

The BER performance at various SNRs for 4-PPM with the four coding schemes is shown in graphical form in fig. 7.1.

Following observations and conclusions are made from the simulation results presented in fig. 7.1:

- BER performance of 4-PPM is the worst with the product code than with any of the other codes for all SNRs considered.

- There is a performance improvement on using it with the RSC code.

- BER performance of 4-PPM is best with the Turbo Code for all SNRs considered.

# 7.2 The 2 x 2 APPM system

The BER performance at various SNRs for 2x2-APPM with the four coding schemes is shown in graphical form in Fig.7.2.

Following observations and conclusions are made from the simulation results presented in fig. 7.2:

- BER performance of 2x2-APPM is the worst with the RSC code than with any of the other codes for all SNRs considered.

- There is a performance improvement on using it with TCM.

- BER performance of 2x2-APPM is the best with Turbo code for all SNRs considered.

7.3 The (5,2) 4-OPPM System

The BER performance at various SNRs for (5,2) 4-OPPM with the four coding schemes is shown in graphical form in Fig.7.3.

Following observations and conclusions are made from the simulation results presented in fig. 7.3:

- BER performance of (5,2)4-OPPM is the worst with the Product code than with any of the other codes for all SNRs considered.

- There is a performance improvement on using it with TCM, and a further improvement on using it with the RSC code.

- BER performance of (5,2) 4-OPPM is the best with Turbo code for all SNRs considered.

7.4 The 4C2 MP-PPM System

The BER performance at various SNRs for 4C2 MP-PPM  with the four coding schemes is shown in graphical form in Fig.7.4.

Following observations and conclusions are made from the simulation results presented in fig. 7.4:

- BER performance of 4C2 MP-PPM is the worst with the  TCM code than with any of the other codes for all SNRs considered.

- There is a performance improvement on using 4C2 MP-PPM  with the product code, and a further improvement on using it with the RSC code.

- BER performance of 4C2 MP-PPM  is the best with Turbo code for all SNRs considered.

# 7.5 Overall Observation and conclusions

The results of the comparative study are given by ranking each coding scheme for a particular PPM scheme, e.g. for the 4-PPM system, Turbo code gives the best performance, followed by RSC code, TCM and Product code. These results have been obtained for SNRs ranging over 0-10 dB. Furthermore,  the duration of each PPM symbol has been chosen equal  to 10-6seconds, for all the PPM schemes. Hence, all simulations have been carried out at a symbol rate of 106 symbols/second.

| PPM system | TCM | Product code | RSC code | Turbo code |
| --- | --- | --- | --- | --- |
| 4-PPM | 3rd | 4th | 2nd | 1st |
| 2x2- APPM | 2nd | 3rd | 4th | 1st |
| (5,2)4-OPPM | 3rd | 4th | 2nd | 1st |
| 4C2 MP-PPM | 4th | 3rd | 2nd | 1st |

Table 7.1 : Relative rankings of different codes for the four PPM schemes

Following observations and conclusions are made from Table 7.1:

- The Turbo code provides the best BER performance over all SNRs considered for all four PPM schemes.

- The relative performance of the other three coding schemes depends on the PPM scheme under consideration.

- For 4-PPM and (5,2) 4-OPPM, the relative performance of the four coding scheme with a lower SNR requirement is preferred as the deep space optical channel is power –constrained. Thus, the Turbo code is ideally suited for this channel.

| PPM system | TCM | Product code | RSC code | Turbo code |
| --- | --- | --- | --- | --- |
| 4-PPM | 7.0 dB | 7.5 dB | 6.5 dB | 2.5 dB |
| 2x2-APPM | >10 dB | >10 dB | >10 dB | 2.5 dB |
| (5,2) 4-OPPM | >10 dB | >10dB | 9.25 dB | 2.5 dB |
| 4C2 MP-PPM | >10 dB | >10 dB | >10 dB | 2.5 dB |

##### SCOPE FOR FUTURE WORK

- In this thesis, four PPM schemes have been considered. An additional PPM scheme called Differential Pulse Position Modulation or DPPM [8] has been seen in the literature and can also be explored.

- In the performance evaluation, Parallel concatenated Convolutional Codes (PCCCs) or Turbo codes have been considered. Other concatenated codes like Serially Concatenated Convolutional Codes (SCCCs) and Hybrid concatenation of Convolutional codes (HCCCs) can also be used to obtain performance improvement.

- It is anticipated that the other powerful coding schemes such as Reed Solomon (RS) codes can achieve better performance, since RS codes can correct both symbol errors as well as symbol erasures which degrade the performance of the communication system substantially. Therefore, the analysis can be extended for RS codes.

- In the thesis, the deep space optical channel has been modeled as an AWGN channel. The simulations can also be carried out for the Poisson photon counting channel.

- In the performance evaluation, perfect symbol synchronization has been assumed, but practically the PPM schemes put substantial burden on the synchronization subsystem which leads to a degraded system performance. Work needs to be carried out keeping in mind the problem of symbol synchronization.

- In this thesis, performance analysis has been carried out for four symbol PPM schemes. A similar analysis can be carried out for larger symbol sets which can transmit more number of bits per channel use, e.g., 16-symbol PPM Schemes for transmitting 4-bits  at a time.

## REFERENCES

[1]	J.H. Franz and V. K. Jain, optical Communication: Componenets and         Systems, Narosa Publishing House, New Delhi,2000.

[2]	M.  Chaudhary,  R. Bose  and  V. K.  Jain,  “ Study of   Trellis   coded         overlapping PPM and Overlapping multi-pulse PPM schemes for deep        space   optical   communication   systems” ,    Seventh     International         Conference on Optoelectronics, FiberOptics & Photonics, Cochin, pp.         LTWP1-10, 9-11 December 2004.

[3] 	R. Bose, Information Theory, Coding and Cryptography, Tata Mc-Graw  Hill, New Delhi, 20002

[4] 	R.  Alves  and   A.  Gameiro,  “  Coding  of  PPM   based   modulation        techniques to improve the  performance  of  infrared  WLANs” ,  IEEE        Vehicular Technology Conference, VTC 2000,  PP,  1345-1352,  vol.3,        24-28 September 2000.

[5] 	B. Sklar ,” A  primer   on   Turbo   code   concepts ”,   Communication       Magazine , IEEE, pp. 94-102, vol. 35, issue 12, December 1997.

[6]	C. Berrou, A. Glavieux and P. Thitimajshima,” Near Shannon Limit error-correcting coding and decoding: Turbo codes”, IEEE Proceedings of the International Conference on Communication, pp, 1064-1070,May 1993, Geneva, Switzerland.

[7]	J.G. Proakis and M. Salehi, communication systems Engineering, Pearson Education Asia, New Delhi, 2002.

[8]	C. Singh, J. John, Y.N. Singh, K.K. Tripathi,’ “A review of indoor optical wireless systems”, IETE Technical Review, pp. 3-17, vol. 19, nos, 1&2, January-April 2002.

[9] 	S. Haykin, Communication Systems, Fourth Edition, John Wiley & Sons Asia, Singapore, 2001.

[10] 	Akshay Batla, post graduate dissertation Error free  Deep space Optical Communication , Deptt. Of Electrical Engineering, IIT, Delhi.

[11]  A. Burr, Turbo codes: the ultimate error correcting codes, Electronics & Communication Engineering Journoul, August 2001.

[12]  D. Divsalar and F. Pollara, Communication Systems Research Section, February, 1995.

[13]  Michael K. Cheng, Bruce E. Moision, Jon Hamkins and Michael A. Nakashima: Implementation of coded Modulation for Deep Space Optical Communications, NASA.

[14]  V. Stankovic & R. Hamzaoui, University of Konstanz

APPENDIX

A guide to the matlab codes in the CD

# Chapter 4

# TCM> PPM

- 4PPM is a file containing the squared-distances between two symbols selected from the 4-PPM constellation, using the |j-l| distance matric.

- 2A2Mppm is a file containing the squared-distances between two symbols selected from the 2x2-APPM constellation, using the Euclidean distance metric.

- 4OPPM is a file containing the squared-distances between two symbols selected from the(5,2) 4-OPPM constellation, using the Manhattan distance Metric.

- 4C2MPppm is a file containing the squared distances between two symbols selected from the 4C2 MP-PPM constellation, using the manhattan distance metric.

- Bitstr_gen is an M-file that generates a random bit stream of length 20. This length 20 is hard coded and canbe changed to suit the requirement.

- Deep_space_1 is an M-file that creates the input 4-PPM symbol stream to the deep space channel, corrupt it with AWGN, then demodulated the corrupted symbol stream appropriately and store it in a file. The input arguments to this M-file are the length of the input bit stream and the SNR respectively.

- Deep_space_2 is an M-file that creates the input 2x2-APPM symbol stream to the deep space channel, corrupt it with AWGN, then demodulated the corrupted symbol stream appropriately and store it in a file. The input arguments to this M-file are the length of the input bit stream and the SNR respectively.

- Deep_space_3 is an M-file that creates the input (5,2) 4-OPPM symbol stream to the deep space channel, corrupt it with AWGN, then demodulated the corrupted symbol stream appropriately and store it in a file. The input arguments to this M-file are the length of the input bit stream and the SNR respectively.

- Deep_space_4 is an M-file that creates the input 4C2 MP-PPM symbol stream to the deep space channel, corrupt it with AWGN, then demodulated the corrupted symbol stream appropriately and store it in a file. The input arguments to this M-file are the length of the input bit stream and the SNR respectively.

- Stat_avg is an M-file that performs statistical averaging while carrying out the simulations. The input arguments to this M-filr are the length of the input bit stream and the SNR respectively.

- Result_gen is an M-file that generates the simulation results in terms of BER over a range of SNR values using the statistical averaging function. The SNR range is hard-coded and can be changed to suit the requirement.

## CHAPTER 5

### PRODUCT CODE-> OOK

- Bitstr_gen is an M-file that generates a random bit stream of length 4. This length 4 is hard coded and canbe changed to suit the requirement.

- LLA is an M-file that performs log-likelihood addition of the input arguments and return the sum.

- Product_code is an M-file that codes the input bit stream to the deep space channel, corrupts it with AWGN, then decode and stores in a file the corrupted bit stream using a specified number of iterations for thesoft input/soft output iterative decoder used. The input arguments to this M-file are the number of decoder iterations and the SNR respectively.

- Stat_avg is an M-file that performs statistical averaging while carrying out the simulations. The input arguments to this M-file are the length of the input bit stream, the number of decoder iterations, the number of iterations for averaging and the SNR respectively.

- Result_gen is an M-file that generates the simulation results in terms of BER over a range of SNR values using the statistical averaging function. The SNR range is hard-coded and can be changed to suit the requirement.

- Uncoded_avg is an M-file that generates the simulation results for the uncoded case. The input arguments to this M-file are the number of iterations for averaging and the SNR respectively.

# Product Code-> PPM

All M-files in this folder have already been explained, except:

- modulate is an M-file that PPM modulates the input coded bit stream and returns the PPM symbol stream thus formed.

- Demodulate is an M-file that demodulates the output symbol stream and returns the bit stream thus formed.

CHAPTER 6

#### RSC CODE->OOK->HOVA

- RSC-encoder is an M-file that codes the input bit stream using the corresponding rate ½ Trellis structure. The input arguments to this M-file are the input bit stream and its length respectively.

- trellis_hard is an M-file that codes the input bit stream to the deep space channel, corrupts it with AWGN, then decoded and stores in a file the corrupted bit stream using the hard Viterbie decoder. The input arguments to this file are the length of the input bit stream and SNR respectively.

- stat_avg is an M-file that performs statistical averaging while carrying out the simulations. The input arguments to this M-file are the length of the input bit stream, the number of iterations for averaging and the SNR respectively.

- Result_gen is an M-file that generates the simulation results in terms of BER over a range of SNR values using the statistical averaging function. The SNR range is hard-coded and can be changed to suit the requirement.

- Uncoded_avg is an M-file that generates the simulation results for the uncoded case. The input arguments to this file are the length of the input bit stream, the number of iterations for averaging and SNR respectively.

RSC Code->OOK->SOVA

All M-files in this folder have already been explained except trellis_soft which involves the use of a soft Viterbie decoder with 3 soft-decision bits to create 8 quantization levels.

# RSC Code->PPM->HOVA

All M-files in this folder have already been explained, except that they have been adjusted for PPM.

##### RSC Code->PPM->SOVA

All M-files in this folder have already been explained, except that they have been adjusted for PPM.

# Turbo Code-> PPM

- int_to_bits is an M-file that converts an integer into a bit vector. The input arguments to this M-file are the integer itself and the length of the bit vector respectively.

- bits_to_int is an M-file that converts a bit vector into a integer. The input argument to this M-file is the bit vector itself.

- Trellis is an M-file that sets up the RSC Trellis structure. The input argument to this M-file is the coefficients of the generator polynomial.

- PCCC_encoder is an M-file that performs the Turbo encoding process. The input arguments to this file are the data burst, the coeff. of the generator polynomial, the random interleaver mapping and puncture bit respectively.

- bit_encoder is an M-file that performs the Turbi encoding process for the single input bit. The input argument to this file are the coeff. of generator polynomial, the input bit itself and the current state of the RSC trellis respectively.

- RSC-encoder is an M-file that performs the role of component RSC encoder for a data burst as a part of the Turbo encoding process. The input arguments to this M-file are the coeff. of the generator polynomial, the data burst and the termination bit respectively.

- Demultiplex is an M-file that performs serial-to-parallel demultiplexing at the receiver to yield the codewords corresponding to each component RSC encoder. The input arguments to this M-file are the received bit stream, the random interleaver mapping and the puncture bit respectively.

- PCCC_decoder is an M-file that performs the Turbo decoding process. The input arguments to this file are the received data burst length, the number of decoder iterations and the number of iterations for averaging respectively.

- Logmap is an M-file that performs the role of the Log-Map component decoder as part of the Turbo decoding process. The input arguments to this file are the received bit stream, the coeff. of the generator polynomial, and a priori information and the index of the current decoder respectively.